Table of Contents

1	Internship Final Report	4

1.1	SBAS Extraction & Hercules Transmitter System	4

1.2	Abstract	4

1.3	Acknowledgments	4

1.4	Table of Contents	4

1.5	Acronyms and Abbreviations	5

1.6	1. Introduction	5

1.6.1	1.1 Problem Statement	6

1.6.2	1.2 Objectives	6

1.6.3	1.3 Deliverables	6

1.7	2. Project Goals and Scope	6

1.7.1	2.1 Goals	6

1.7.2	2.2 Scope	7

1.7.3	2.3 Non‑Functional Requirements	7

1.7.4	2.4 Constraints and Assumptions	7

1.8	3. Background: SBAS and RINEX	7

1.8.1	3.1 SBAS Message Structure (Summary)	7

1.8.2	3.2 RINEX SBAS Broadcast Records	8

1.8.3	3.3 Why Typed Message Structures Matter	8

1.8.4	3.4 Integrity and Safety Considerations	8

1.8.5	3.5 SBAS Timing Model	8

1.8.6	3.6 Practical Implications for Implementation	8

1.9	4. System Overview	8

1.10	5. Architecture and Data Flow	9

1.10.1	5.1 High‑Level Architecture	9

1.10.2	5.2 Data Flow (End‑to‑End)	9

1.10.3	5.3 Architecture Diagram (Conceptual)	9

1.10.4	5.4 Runtime Loop Summary	10

1.10.5	5.5 Scheduling Logic (Operational View)	10

1.10.6	5.6 Message Editing Flow	10

1.11	6. Backend Implementation (C)	10

1.11.1	6.1 Core Modules	10

1.11.2	6.2 Data Structures and Message Model	11

1.11.3	6.3 Detailed Module Walkthrough	12

1.11.4	6.4 RINEX Parsing Pipeline (Detailed)	13

1.11.5	6.5 CRC‑24Q Validation	14

1.11.6	6.6 Message Decoding	14

1.11.7	6.7 In‑Memory Storage (Message Array)	14

1.11.8	6.8 Transmission Scheduler	14

1.11.9	6.9 Editing and Increment Rules	14

1.11.10	6.10 Encoding and Broadcast	15

1.11.11	6.11 Algorithm Details	15

1.11.12	6.12 Message Type Implementation Notes (Selected Examples)	15

1.11.13	6.13 Orbit Models	16

1.11.14	6.14 RINEX Output Handling	16

1.11.15	6.15 IOD Synchronization	17

1.11.16	6.16 Scheduler Priorities	17

1.11.17	6.17 Error Handling and Validation	17

1.11.18	6.18 Memory Management	17

1.11.19	6.10 Orbit Models	17

1.11.20	6.11 RINEX Output Writer	17

1.12	7. Frontend Implementation (PyQt6 GUI)	18

1.12.1	7.1 Presentation Layer (Main Window)	18

1.12.2	7.2 Field Editor Widget	18

1.12.3	7.3 Controller Layer	18

1.12.4	7.4 User Workflows	19

1.12.5	7.5 GUI Architecture Diagram	19

1.12.6	7.6 State Management	19

1.12.7	7.7 Logging and Status Updates	19

1.12.8	7.8 Console Menu Parity	19

1.12.9	7.5 GUI Error Handling	20

1.12.10	7.4 API Wrapper Layer	20

1.13	8. DLL Integration Layer	20

1.13.1	8.1 API Categories	20

1.13.2	8.2 JSON Message Format	21

1.13.3	8.3 Status Callbacks	21

1.14	9. Build, Deployment, and Runtime Workflow	21

1.14.1	9.1 Build Process	21

1.14.2	9.2 Environment and Dependencies	21

1.14.3	9.2 Runtime Workflow	21

1.14.4	9.3 Operational Profiles	22

1.14.5	9.4 End‑to‑End Example Walkthrough	22

1.15	10. Testing and Validation	23

1.15.1	10.1 Unit and Integration Tests	23

1.15.2	10.2 Manual Validation	23

1.15.3	10.3 Test Scenarios	23

1.15.4	10.4 Observations	23

1.16	11. Performance and Reliability Considerations	23

1.16.1	11.1 Time Complexity	24

1.16.2	11.2 Memory Usage	24

1.16.3	11.3 Reliability Features	24

1.16.4	11.4 Security Considerations	24

1.17	12. Challenges and Resolutions	24

1.17.1	Challenge 1: Parsing RINEX Variants	24

1.17.2	Challenge 2: Correct Bit‑Level Encoding	24

1.17.3	Challenge 3: GUI Responsiveness	25

1.17.4	Challenge 4: Maintaining Feature Parity	25

1.17.5	Challenge 5: Orbit Model Integration	25

1.17.6	Challenge 6: Usability of Complex Fields	25

1.18	13. Future Enhancements	25

1.19	14. Conclusion	25

1.20	15. References	26

1.21	16. Appendix A: Public DLL API Summary	26

1.21.1	16.1 API Reference (Functional Summary)	26

1.22	17. Appendix B: Message Types and Update Intervals	27

1.23	18. Appendix C: Project Directory Structure	28

1.24	19. Appendix D: Message Field Reference (Summary)	28

1.25	20. Appendix E: Operational Checklist	29

# 1	Internship Final Report

## 1.1	SBAS Extraction & Hercules Transmitter System

Candidate: Abdul Rauf
Organization: NASTP 
Supervisor: Mam Maria, Mam Nimra
Internship Period: 25 august 25 – 25 january 26
Location: Rawalpini
Date of Submission: 5/2/2026

## 1.2	Abstract

This report documents the internship work completed on the SBAS Extraction and Hercules Transmitter System. The project delivers a complete, end‑to‑end pipeline for parsing RINEX SBAS broadcast files, decoding SBAS message types into typed C structures, editing and re‑encoding messages, and transmitting them over TCP. A professional GUI built with PyQt6 provides a user‑friendly interface for file loading, live transmission control, message editing, and configuration of orbit models and RINEX output. The system integrates a C‑based backend (performance‑critical parsing, scheduling, and encoding) with a Python GUI front end via a DLL and ctypes bindings. This report provides a clear, professional, and detailed description of the full implementation, architecture, data flow, algorithms, and testing results.

## 1.3	Acknowledgments

I would like to thank my supervisor and the engineering team for their guidance and support during this internship. Their expertise in SBAS standards, RINEX processing, and system design was invaluable to my learning and to the success of the project.

## 1.4	Table of Contents

- Introduction

- Project Goals and Scope

- Background: SBAS and RINEX

- System Overview

- Architecture and Data Flow

- Backend Implementation (C)

- Frontend Implementation (PyQt6 GUI)

- DLL Integration Layer

- Build, Deployment, and Runtime Workflow

- Testing and Validation

- Performance and Reliability Considerations

- Challenges and Resolutions

- Future Enhancements

- Conclusion

- References

- Appendix A: Public DLL API Summary

- Appendix B: Message Types and Update Intervals

- Appendix C: Project Directory Structure

- Appendix D: Message Field Reference (Summary)

- Appendix E: Operational Checklist

## 1.5	Acronyms and Abbreviations

- SBAS: Satellite‑Based Augmentation System

- GNSS: Global Navigation Satellite System

- RINEX: Receiver INdependent EXchange format

- MT: Message Type

- IODP: Issue of Data PRN mask

- IODF: Issue of Data Fast Corrections

- IODI: Issue of Data Ionospheric

- UDREI: User Differential Range Error Index

- GIVEI: Grid Ionospheric Vertical Error Index

- CRC: Cyclic Redundancy Check

- TLE: Two‑Line Element set

- ECEF: Earth‑Centered, Earth‑Fixed coordinate system

- ECI: Earth‑Centered Inertial coordinate system

- GMST: Greenwich Mean Sidereal Time

- DLL: Dynamic Link Library

- API: Application Programming Interface

- UI: User Interface

## 1.6	1. Introduction

The SBAS Extraction and Hercules Transmitter System is a complete toolchain for decoding, editing, and broadcasting Satellite‑Based Augmentation System (SBAS) correction messages. SBAS systems such as WAAS, EGNOS, MSAS, and GAGAN transmit augmentation data used by GNSS receivers to improve accuracy and integrity. These messages are distributed in RINEX broadcast files and, in live systems, are transmitted continuously to clients.

This internship focused on implementing and documenting a full‑stack system that: - Reads and parses RINEX SBAS broadcast files. - Validates and decodes SBAS message payloads into typed structures. - Enables interactive editing of message fields. - Re‑encodes messages and broadcasts them on a TCP server. - Provides a modern graphical interface for usability and operational efficiency.

The result is a production‑ready framework suitable for research, testing, and simulation of SBAS data streams.

### 1.6.1	1.1 Problem Statement

Organizations that research or validate SBAS performance often need to replay, edit, or simulate correction messages at scale. Traditional tooling is fragmented: RINEX parsers are separate from network broadcasters, and many tools provide only partial access to raw message fields. The absence of a unified pipeline makes verification, experimentation, and demonstration workflows slow and error‑prone.

This project addresses that gap by providing a single integrated system that accepts RINEX broadcast data, decodes it into structured message models, allows deterministic edits, and transmits messages with correct timing and integrity behavior.

### 1.6.2	1.2 Objectives

- Provide a trustworthy, bit‑accurate SBAS pipeline from file to network.

- Maintain strict adherence to SBAS message layouts and update intervals.

- Enable structured field editing with automatic increment behavior to simulate realistic changes.

- Offer both console and GUI interfaces for different user needs.

### 1.6.3	1.3 Deliverables

- C backend libraries for parsing, decoding, scheduling, and encoding.

- Hercules Transmitter console application.

- DLL interface for GUI integration.

- PyQt6 GUI with transmission, editing, and configuration features.

- Professional documentation and test utilities.

## 1.7	2. Project Goals and Scope

### 1.7.1	2.1 Goals

- Implement a complete SBAS message processing pipeline.

- Provide full support for SBAS message types (MT1–MT28, MT62, MT63).

- Implement transmission scheduling according to RTCA DO‑229D intervals.

- Enable user‑driven message edits with automatic increment rules.

- Provide orbit model overrides (TLE and Statistical models).

- Build a professional GUI for operations and message editing.

- Generate RINEX output files matching input templates.

- Provide clear documentation for future maintainers.

### 1.7.2	2.2 Scope

The scope includes both backend and frontend development: - Backend (C): parsing, decoding, encoding, scheduling, networking, and data management. - Frontend (Python/PyQt6): user interface, configuration, visualization, and control logic. - Integration: DLL API and ctypes wrapper.

### 1.7.3	2.3 Non‑Functional Requirements

- Performance: Support real‑time transmission with 1‑second scheduling granularity.

- Correctness: Preserve bit‑level accuracy for SBAS message encoding/decoding.

- Reliability: Handle malformed packets without crashing; reject invalid CRCs.

- Usability: Provide a GUI that mirrors console functionality.

- Maintainability: Modular design with well‑defined interfaces between layers.

### 1.7.4	2.4 Constraints and Assumptions

- Input files conform to RINEX 2.10 SBAS broadcast conventions.

- Target platforms include Windows (primary) and Unix‑like systems (secondary).

- TCP transmission is intended for local or trusted networks.

- Maximum client count is limited by server configuration.

## 1.8	3. Background: SBAS and RINEX

SBAS systems broadcast augmentation data to improve GNSS accuracy and integrity. The messages follow a standardized format that defines payload structures, update intervals, and integrity indicators. RINEX 2.10 SBAS broadcast files store these messages as hexadecimal payloads with timestamps and satellite identifiers.

Key concepts: - Message Types (MT): Each message type has a unique payload structure (e.g., MT1 for PRN mask, MT2‑5 for fast corrections). - Integrity Indicators: UDREI and GIVEI fields provide confidence levels for corrections. - Issue of Data (IOD): Fields such as IODP, IODF, and IODI link related message groups. - RINEX: A standardized, text‑based file format for GNSS data.

The implementation adheres to RTCA DO‑229D intervals and SBAS message layouts, ensuring that the simulated data stream behaves like a real SBAS broadcaster.

### 1.8.1	3.1 SBAS Message Structure (Summary)

An SBAS frame is 250 bits long and contains: - Preamble (8 bits): Synchronization byte. - Message Type (6 bits): Identifies the payload structure. - Payload (212 bits): Message‑specific fields. - CRC (24 bits): CRC‑24Q error check.

### 1.8.2	3.2 RINEX SBAS Broadcast Records

RINEX SBAS broadcast data appears after the header section and is organized in lines that include: - Satellite identifiers (PRN, GEO ID) - Timestamp fields (year, month, day, hour, minute, second) - Hexadecimal payload data across one or more lines

The system must assemble these lines into complete packets and validate them before decoding.

### 1.8.3	3.3 Why Typed Message Structures Matter

SBAS messages carry many fields with different bit widths and scales. Using typed structures ensures that: - Every field is accessed and modified in a consistent, safe manner. - Encoders can reconstruct frames exactly as specified by the standard. - GUI editing can map fields directly to user inputs without manual bit arithmetic.

### 1.8.4	3.4 Integrity and Safety Considerations

SBAS is a safety‑critical domain. Even in simulation or test environments, it is essential to preserve: - Correct timing (update intervals) - Valid integrity indicators (UDREI/GIVEI) - Consistent IOD fields across message sets

The system enforces these constraints through validation, scheduling rules, and synchronized IOD handling.

### 1.8.5	3.5 SBAS Timing Model

SBAS messages are not transmitted arbitrarily; each message type has a maximum update interval. For example: - Fast corrections (MT2‑MT5) must be broadcast every 6 seconds. - GEO navigation (MT9) must be refreshed every 120 seconds. - Ionospheric corrections (MT26) are updated every 300 seconds.

The transmitter’s scheduler uses these intervals to produce a realistic broadcast stream. If no message is due, MT63 filler messages are transmitted to maintain continuity.

### 1.8.6	3.6 Practical Implications for Implementation

The timing and integrity requirements translate into concrete engineering choices: - A deterministic 1‑second scheduler loop. - Priority handling for integrity‑critical message types. - Auto‑increment rules that preserve monotonic change in edited fields. - Consistent IOD propagation across related message families.

## 1.9	4. System Overview

The system consists of three major layers:

- Backend (C Core)

  - Parsing: Reads and validates RINEX packets.

  - Decoding: Converts payloads to typed SBAS message structures.

  - Scheduling: Determines which message to transmit next.

  - Encoding: Rebuilds messages into 250‑bit SBAS frames.

  - Networking: Broadcasts messages to TCP clients.

- DLL API Layer

  - Exposes backend functionality via a C API.

  - Provides initialization, binding, transmission, editing, and configuration functions.

- Frontend (PyQt6 GUI)

  - Provides a user interface for file loading, transmission control, and field editing.

  - Uses the DLL via ctypes for direct communication with the backend.

The design ensures both operational usability (GUI) and engineering flexibility (console + DLL API).

## 1.10	5. Architecture and Data Flow

### 1.10.1	5.1 High‑Level Architecture

The architecture is a layered system with clean separation of concerns:

- Presentation Layer (PyQt6): Handles UI interactions.

- Controller Layer: Manages application state and threading.

- API Wrapper (ctypes): Exposes DLL functions to Python.

- C DLL (Backend Core): Implements the full SBAS pipeline.

### 1.10.2	5.2 Data Flow (End‑to‑End)

- User selects a RINEX SBAS file.

- Backend parses file and extracts SBAS packets.

- Each packet is CRC‑validated and decoded into typed structures.

- Messages are stored in memory for fast access.

- Scheduler determines which message is due for transmission.

- Encoder builds the binary SBAS frame and computes CRC.

- Message is broadcast to connected TCP clients.

- Optional: message is logged to a RINEX output file.

### 1.10.3	5.3 Architecture Diagram (Conceptual)

┌──────────────────────────────────────────────┐
│                 PyQt6 GUI                   │
│  - Transmission control                      │
│  - Message field editor                      │
│  - Configuration & output settings           │
└───────────────┬──────────────────────────────┘
                │ ctypes
┌───────────────▼──────────────────────────────┐
│           hercules_transmitter.dll           │
│  - C API wrappers                            │
│  - Scheduler & transmission                  │
│  - Message editing & encoding                │
└───────────────┬──────────────────────────────┘
                │ internal calls
┌───────────────▼──────────────────────────────┐
│                 Backend Core                 │
│  RINEX Parser → Decoder → Scheduler → TCP    │
│  Orbit Models → Encoder → RINEX Writer       │
└──────────────────────────────────────────────┘

### 1.10.4	5.4 Runtime Loop Summary

The transmission loop runs at a fixed cadence (1 second), mirroring the SBAS timing resolution. Each step: 1. Accepts new TCP clients (non‑blocking). 2. Determines which message type is overdue. 3. Applies increment rules if the message was edited. 4. Encodes and broadcasts the message. 5. Logs to RINEX output if enabled.

### 1.10.5	5.5 Scheduling Logic (Operational View)

The scheduler operates on a per‑second tick. For each tick: 1. Evaluate all supported message types. 2. Determine if each type is overdue (current_time - last_sent > interval). 3. Select the most overdue message type (priority to integrity types). 4. Encode and broadcast the selected message. 5. If none are due, send MT63 filler.

### 1.10.6	5.6 Message Editing Flow

- GUI loads a message in JSON form from the DLL.

- User edits fields in the grid.

- GUI sends updated values back to the DLL.

- DLL updates the in‑memory typed message.

- Auto‑increment rules are enabled for continuous sends.

## 1.11	6. Backend Implementation (C)

The backend is the core of the system, implemented in C for performance and low‑level control. It is responsible for parsing, decoding, encoding, scheduling, and broadcasting SBAS messages.

### 1.11.1	6.1 Core Modules

File_Parsing.c - Implements the main pipeline. - Converts timestamps to Unix time (utility present; header storage disabled in current code). - Performs CRC validation and message type filtering. - Creates the in‑memory message array.

rinex_reader.c - Parses the RINEX file content. - Detects SBAS packet lines and assembles complete packets. - Extracts timestamps and satellite IDs. - Produces SBAS_Packet entries for decoding.

message_types.c - Implements decoders for all supported SBAS message types. - Extracts fields with MSB‑first bit slicing. - Populates typed message structures.

encoder.c - Implements encoders to build SBAS frames from typed structures. - Uses a common bit‑writer and CRC‑24Q generator. - Provides MT2‑5 wrappers and per‑type encoding functions.

tx_source.c - Manages message storage and lookup. - Provides fast search by message type and satellite ID. - Supplies messages to scheduler for transmission.

tx_server.c - Implements TCP server using Winsock. - Accepts clients non‑blocking. - Broadcasts messages to all connected clients. - Handles disconnects and cleans up resources.

hercules_transmitter.c - Console application providing an interactive menu. - Implements scheduler, editor, and broadcast loop.

hercules_api.c - DLL interface exposing backend capabilities. - Mirrors console behavior for GUI integration.

### 1.11.2	6.2 Data Structures and Message Model

All SBAS message types are defined as typed structs in sbas_message_structs.h. Each struct stores raw field values in their correct bit sizes. A union‑based SBAS_TypedMessage holds any message type, allowing: - Type‑safe access to fields. - Consistent encoding and decoding. - Clean mapping to GUI editing features.

#### 1.11.2.1	6.2.1 Core Structure Summary

| Structure | Purpose | Key Fields |
| --- | --- | --- |
| SBAS_MessageHeader | Common header for SBAS messages | preamble, message_type, satellite_id |
| SBAS_TypedMessage | Union wrapper for all MTs | message_type, satellite_id, data union |
| SBAS_TimestampMessagePair | Storage element in message array | timestamp, message |
| SBAS_MessageArray | Dynamic array of message pairs | pairs, count, capacity |

These structures establish a consistent representation used across parsing, decoding, editing, and encoding stages.

### 1.11.3	6.3 Detailed Module Walkthrough

#### 1.11.3.1	6.3.1 File_Parsing.c (Pipeline Core)

Responsibilities: - Owns the pipeline from raw RINEX text to typed message arrays. - Validates CRC‑24Q before decoding. - Handles satellite ID and message type filtering.

Key behaviors: - Converts timestamps using a Unix timestamp utility. This conversion is implemented and ready for use; header storage is currently disabled in code, making timestamps optional in the stored header. - Uses dynamic growth for message storage to avoid reallocating on each append. - Ensures correctness by rejecting corrupted packets early.

#### 1.11.3.2	6.3.2 rinex_reader.c (RINEX Parsing)

Responsibilities: - Tokenizes lines after the RINEX header. - Detects SBAS packet lines and parses satellite ID and timestamp. - Assembles full hex payloads across multiple lines.

Key implementation points: - Portable tokenization using compatibility shims for Windows. - Identification of SBAS packet lines through numeric patterns and field positions. - Flexible parsing to handle minor variations in file formatting.

#### 1.11.3.3	6.3.3 message_types.c (Decoding)

Responsibilities: - Provides decoding functions for all supported message types. - Extracts bit‑accurate fields for each message structure. - Produces human‑readable summaries for tests.

Key implementation points: - Uses MSB‑first bit extraction helpers (sbas_bits_get_u64/s64). - Maintains strict offsets for each field as per SBAS ICD. - Supports into‑struct decoding for structured access in later stages.

#### 1.11.3.4	6.3.4 encoder.c (Encoding)

Responsibilities: - Encodes typed structures into SBAS frames. - Writes header and payload bits to a byte buffer. - Computes CRC‑24Q and appends it to the output.

Key implementation points: - Unified MT2‑5 encoding wrapper reduces duplication. - Consistent bit‑writer ensures exact alignment of payload fields. - Encoders are symmetric with decoders to preserve data integrity.

#### 1.11.3.5	6.3.5 tx_source.c (Message Source Provider)

Responsibilities: - Owns the in‑memory message array. - Provides search and iteration by message type and satellite ID. - Supplies messages to the scheduler for transmission.

Key implementation points: - Provides functions for next message cycling by type. - Tracks bound file path for reference and logging. - Exposes message count and indexed access.

#### 1.11.3.6	6.3.6 tx_server.c (TCP Broadcast Layer)

Responsibilities: - Manages a TCP server socket. - Accepts and tracks multiple clients. - Broadcasts binary SBAS frames to all connected clients.

Key implementation points: - Uses non‑blocking sockets to avoid stalls. - Handles disconnect detection and cleanup. - Designed for real‑time continuous streaming.

#### 1.11.3.7	6.3.7 hercules_transmitter.c (Console Interface)

Responsibilities: - Provides menu‑driven console operations. - Implements direct scheduling and manual send features. - Manages edit‑and‑send workflow for message fields.

Key implementation points: - Works with the same core modules used by the DLL. - Provides auto‑increment behavior identical to DLL mode. - Includes default TCP port (23) for telnet‑friendly access.

#### 1.11.3.8	6.3.8 hercules_api.c (DLL API)

Responsibilities: - Exposes backend functionality for the GUI. - Provides structured JSON output for message fields. - Handles message updates and single‑message transmission.

Key implementation points: - Maintains state for transmission, scheduling, and single‑message mode. - Implements status callbacks to notify GUI. - Mirrors console functionality for feature parity.

### 1.11.4	6.4 RINEX Parsing Pipeline (Detailed)

Step 1: Read File - The file is loaded into memory as a byte buffer.

Step 2: Identify RINEX Format - The parser detects whether the file is in RINEX 2 format based on filename and header markers.

Step 3: Tokenize and Parse - Each line is tokenized. - SBAS lines are detected by the presence of SBAS markers and numeric fields.

Step 4: Packet Assembly - Hex fragments are collected across multiple lines. - Timestamps and satellite IDs are associated with the packet.

Step 5: Validation - CRC‑24Q is computed and verified. - Message type and satellite ranges are checked.

Step 6: Decode to Typed Message - Each packet is decoded into a typed SBAS structure. - The typed message is appended to the message array.

This pipeline ensures that only valid SBAS messages reach the transmission stage.

### 1.11.5	6.5 CRC‑24Q Validation

The system implements CRC‑24Q exactly as defined in SBAS specifications: - Polynomial: 0x1864CFB - MSB‑first bit processing - 24‑bit register

CRC is computed over bits 0–225 and compared against the trailing 24‑bit CRC field. Invalid messages are rejected early to ensure data integrity.

### 1.11.6	6.6 Message Decoding

Each message type has a dedicated decode function. These functions: - Compact input hex. - Extract fields using MSB‑first bit slicing. - Store results in typed structures. - Preserve raw integer values for correct re‑encoding.

Supported message types include: - MT1: PRN mask - MT2–MT5: Fast corrections - MT6: Integrity information - MT7: Degradation parameters - MT9: GEO navigation - MT10: Degradation parameters - MT12: UTC parameters - MT17: GEO almanac - MT18: Ionospheric grid mask - MT24: Mixed corrections - MT25: Long‑term corrections - MT26: Ionospheric delay - MT28: Covariance matrix - MT62/63: Internal or filler messages

### 1.11.7	6.7 In‑Memory Storage (Message Array)

Parsed messages are stored in a dynamic array: - Each entry contains a timestamp and a typed message. - Growth strategy uses 1.5x expansion for memory efficiency. - Stored data supports fast lookup by type and satellite.

This design provides: - Efficient access during scheduling. - Minimal parsing overhead after file load.

### 1.11.8	6.8 Transmission Scheduler

The scheduler in hercules_transmitter.c is a direct scheduler that: - Tracks last‑sent timestamps for each message type. - Compares update intervals against the current time. - Selects the most overdue message type. - Prioritizes integrity‑critical messages (MT2‑6, MT24). - Sends MT63 filler messages when no data is due.

Update intervals follow RTCA DO‑229D Table A‑25 (see Appendix B).

### 1.11.9	6.9 Editing and Increment Rules

Editing is supported for all key message types. The system records which fields were changed, then applies an auto‑increment rule starting from the second transmission. This preserves the user’s exact values for the first send and simulates realistic variation afterward.

Examples: - MT9 position values increment by their on‑wire LSB each cycle. - IOD values wrap correctly within their bit ranges. - Fast correction arrays increment consistently to maintain block coherence.

### 1.11.10	6.10 Encoding and Broadcast

The encoder module reconstructs SBAS messages into 250‑bit frames by: - Writing header bits (preamble and message type). - Writing payload fields in correct bit order. - Computing CRC‑24Q. - Producing a hex string that can be converted to bytes.

The broadcast module (tx_server.c): - Uses Winsock on Windows. - Maintains a list of connected clients. - Sends data to all clients using non‑blocking sockets. - Handles disconnects gracefully.

### 1.11.11	6.11 Algorithm Details

#### 1.11.11.1	6.11.1 CRC‑24Q Algorithm

The system implements CRC‑24Q bitwise with MSB‑first traversal. For each bit, the register shifts left and applies the polynomial if the feedback bit is set. This matches SBAS requirements and ensures accurate validation of SBAS frames.

#### 1.11.11.2	6.11.2 Dynamic Array Growth

Message arrays use exponential growth (1.5x or 2x, depending on context) for amortized efficiency. This avoids repeated allocations while handling large RINEX datasets.

#### 1.11.11.3	6.11.3 Direct Scheduler Selection

The scheduler calculates overdue time for each message type as: current_time - last_sent[type] - interval[type] The message with the largest positive overdue value is selected. Integrity‑critical types receive higher priority during tie‑breaks.

#### 1.11.11.4	6.11.4 Auto‑Increment Rules

For edited messages, the system applies a change‑tracking rule: - First transmission after edit uses exact user values. - Subsequent transmissions increment those fields by their LSB step size. - Values wrap or clamp to valid ranges, preserving field validity.

#### 1.11.11.5	6.11.5 TLE Propagation (Two‑Body Model)

The TLE module uses a simplified two‑body Keplerian model: - Parses epoch and orbital elements from TLE lines. - Converts epoch to Julian Date. - Solves Kepler’s equation for eccentric anomaly. - Computes ECI position/velocity. - Transforms to ECEF using GMST and Earth rotation rate.

This approach is adequate for GEO satellites and short‑term simulations.

### 1.11.12	6.12 Message Type Implementation Notes (Selected Examples)

The following notes summarize how key message types are represented and encoded. Each message’s typed struct mirrors the SBAS ICD bit layout. Values are stored in raw integer form (LSB‑aligned) to preserve fidelity.

MT1 – PRN Mask - 210‑bit PRN mask stored as a packed 27‑byte array. - IODP stored as a 2‑bit field.

MT2–MT5 – Fast Corrections - IODF and IODP are 2‑bit fields. - 13 fast correction values, each stored as signed 12‑bit values. - 13 UDREI values, each 4 bits.

MT6 – Integrity Information - IODF2..IODF5 (2 bits each). - UDREI array for 51 satellites.

MT7 – Degradation Parameters - t_lat (4 bits), iodp (2 bits), and UDRE AI array.

MT9 – GEO Navigation - Contains t0, URA, position, velocity, acceleration, and clock terms. - Encoding uses different LSB scales per field (e.g., X/Y position at 0.08 m, Z at 0.4 m).

MT10 – Degradation Parameters - Encodes multiple coefficients and intervals with different scales. - Contains two 1‑bit flags (rss_udre, rss_iono) and spare bits.

MT17 – GEO Almanac - Includes three GEO blocks, each with position and velocity terms. - t0 reference time stored at the end of the payload.

MT18 – IGP Mask - number_of_bands, band_number, and iodi followed by a 201‑bit mask.

MT24 – Mixed Corrections - Includes 6 fast corrections and UDREIs, plus long‑term correction fields.

MT25 – Long‑Term Corrections - Two halves per message, each with velocity_code and either two‑sat or single‑sat with rates. - Supports both v0 (two‑sat) and v1 (single‑sat with rates) layouts.

MT26 – Ionospheric Delay - 15 IGP entries with vertical delay and GIVEI fields.

MT28 – Covariance Matrix - Two satellite blocks with diagonal and off‑diagonal covariance values.

### 1.11.13	6.13 Orbit Models

The system supports orbit model overrides: - TLE Model: Parses standard Two‑Line Elements and uses two‑body propagation for GEO prediction. Converts ECI to ECEF using GMST and applies Earth rotation compensation. - Statistical Models (128/143): Learns movement patterns from historical MT9 data and predicts future positions using averaged deltas and rate patterns. - Simulation Mode: Optional fixed time base for reproducible test runs.

### 1.11.14	6.14 RINEX Output Handling

The RINEX output writer uses the input file header as a template. The output flow is: 1. User selects an output path in the GUI. 2. The DLL initializes the writer using the input file as a template. 3. Every transmitted message is appended with updated timestamps. 4. MT63 fillers are also logged, ensuring continuity.

This approach provides a valid, end‑to‑end output dataset that can be re‑loaded for future analysis or replay.

### 1.11.15	6.15 IOD Synchronization

SBAS message families require consistent Issue of Data fields across related message types. The system supports global overrides: - IODP: synchronized across MT1, MT2‑5, MT7, MT24, MT25, MT28. - IODF: synchronized across MT2‑5, MT6, MT24. - IODI: synchronized across MT18 and MT26.

When enabled, edits to an IOD field propagate automatically so that related messages remain internally consistent.

### 1.11.16	6.16 Scheduler Priorities

To protect integrity, the scheduler assigns higher priority to: - MT2‑MT6 (fast corrections and integrity) - MT24 (mixed corrections)

If multiple message types are overdue, these types are preferred even if their overdue time is slightly lower than others. This behavior aligns with operational SBAS safety priorities.

### 1.11.17	6.17 Error Handling and Validation

The backend includes: - Parameter validation for all public APIs. - Null checks on buffers and pointers. - Error status strings returned via DLL calls. - CRC validation and type/satellite filters for all parsed packets.

### 1.11.18	6.18 Memory Management

- Message arrays are dynamically allocated and freed through a controlled pipeline.

- Bound file data is released when unbinding.

- TCP sockets and client arrays are cleaned up on shutdown.

### 1.11.19	6.10 Orbit Models

The system supports orbit model overrides: - TLE Model: Parses standard Two‑Line Elements and uses two‑body propagation for GEO prediction. Converts ECI to ECEF using GMST and applies Earth rotation compensation. - Statistical Models (128/143): Learns movement patterns from historical MT9 data and predicts future positions using averaged deltas and rate patterns. - Simulation Mode: Optional fixed time base for reproducible test runs.

These models allow controlled test scenarios and realistic motion simulation.

### 1.11.20	6.11 RINEX Output Writer

The backend can optionally log transmitted messages to a RINEX output file: - Uses the input file header as a template. - Updates timestamps dynamically. - Records all transmitted packets, including MT63 fillers.

This provides a reproducible output dataset aligned with the input format.

## 1.12	7. Frontend Implementation (PyQt6 GUI)

The GUI provides a professional, user‑friendly interface for operating the system. It is implemented in Python with PyQt6 and follows a clean three‑layer design.

### 1.12.1	7.1 Presentation Layer (Main Window)

The GUI provides three primary tabs:

Transmission Tab - File browser for RINEX input. - Satellite filter selector. - Fast‑correction satellite count. - Transmission duration control. - Start/Stop buttons. - Real‑time client and message counters.

Message Editor Tab - Satellite ID selector. - Message type dropdown. - Field editor with structured inputs. - Send‑once and continuous transmission controls.

Configuration Tab - Orbit model selection (None, TLE, Statistical 128/143). - RINEX output configuration.

The main window uses a consistent layout with labeled controls, a log panel, and a status bar. A timer refreshes the status (clients, messages, elapsed time) every second. This ensures operational clarity and immediate feedback.

### 1.12.2	7.2 Field Editor Widget

The field editor provides a structured grid that: - Flattens nested JSON data into dotted field names. - Displays editable input boxes per field. - Tracks modified fields and highlights changes. - Applies updates in batch using the DLL API. - Enables auto‑increment mode after edits.

This makes editing complex SBAS message structures approachable for users.

Field editor design highlights: - Uses a scrollable grid to handle large message structures. - Supports list indices with natural sorting (e.g., field[2] after field[1]). - Provides “Update” and “Reset” actions for safe editing. - Displays a header indicating the active satellite and message type.

### 1.12.3	7.3 Controller Layer

The controller manages: - Initialization and error handling. - Background transmission loop in a separate thread. - Logging to the GUI. - Synchronization of state between UI and backend.

A 1‑second timing loop mirrors the console scheduler and ensures consistent message timing.

Controller responsibilities include: - Calling the DLL initialization routine and handling failures. - Starting and stopping transmission in a controlled manner. - Enforcing that only one transmission thread runs at a time. - Capturing and displaying errors without crashing the GUI.

### 1.12.4	7.4 User Workflows

Workflow A: Quick Broadcast 1. Launch GUI. 2. Load a RINEX SBAS file. 3. Start transmission with default settings. 4. Connect a TCP client to view messages.

Workflow B: Message Editing 1. Load file and open Message Editor tab. 2. Select satellite ID and message type. 3. Edit fields and press Update. 4. Send once or start continuous transmission.

Workflow C: Orbit Model Simulation 1. Load file. 2. Apply TLE or Statistical model. 3. Start transmission and observe predicted motion.

### 1.12.5	7.5 GUI Architecture Diagram

MainWindow (QMainWindow)
 ├── Transmission Tab
 │   ├── File Loader
 │   ├── Filter Settings
 │   └── Start/Stop Controls
 ├── Message Editor Tab
 │   ├── Selector (Satellite, MT)
 │   └── FieldEditorWidget
 ├── Configuration Tab
 │   ├── Orbit Model Selector
 │   └── RINEX Output Controls
 └── System Log Panel

### 1.12.6	7.6 State Management

The controller maintains application state, including: - is_initialized - is_transmitting - is_continuous_tx - transmission_thread

State changes are reflected in UI elements such as button enable/disable states and status text.

### 1.12.7	7.7 Logging and Status Updates

The GUI maintains a system log panel to capture: - Initialization events - File binding results - Transmission start/stop events - Errors and warnings

Status updates are refreshed every second to show: - Client count - Message count - Transmission time elapsed and remaining

### 1.12.8	7.8 Console Menu Parity

The GUI was designed to mirror the console menu options: 1. Start Transmission → Transmission tab (Start button) 2. Stop Transmission → Transmission tab (Stop button) 3. Show Client Status → Status panel (client count) 4. Edit and Send Message → Message Editor tab (Send Once) 5. Set Orbit Model → Configuration tab (Orbit model section) 6. Edit Single Message Continuously → Message Editor tab (Start Continuous) 7. Configure RINEX Output → Configuration tab (RINEX output section)

This parity ensures that experienced console users can transition seamlessly to the GUI.

### 1.12.9	7.5 GUI Error Handling

- DLL errors are converted into Python exceptions and shown in dialogs.

- User input fields include basic validation (type ranges in spin boxes).

- Missing DLL or file issues are displayed via QMessageBox.

### 1.12.10	7.4 API Wrapper Layer

The Python wrapper uses ctypes to: - Load hercules_transmitter.dll. - Define argument and return types for each exported C function. - Provide Pythonic methods with exception handling.

This layer bridges high‑level GUI actions with low‑level C implementation.

## 1.13	8. DLL Integration Layer

The DLL API exposes a complete set of functions for GUI integration:

- Initialization and cleanup

- File binding

- Transmission control

- Message retrieval and field updates

- Single‑message transmission

- Orbit model configuration

- RINEX output control

- Status and logging callbacks

The DLL mirrors the console application behavior, ensuring feature parity.

### 1.13.1	8.1 API Categories

- Initialization: Create and tear down the core system and TCP server.

- File Binding: Load and parse a RINEX file as the message source.

- Transmission Control: Start, stop, and step the scheduler loop.

- Message Editing: Retrieve structured fields and update values.

- Single‑Message Modes: Send once or continuous.

- Orbit Models: Apply TLE or Statistical predictions.

- RINEX Output: Enable/disable output writer with template binding.

### 1.13.2	8.2 JSON Message Format

The DLL returns message data as JSON to simplify GUI rendering. The JSON includes: - satellite_id - message_type - fields object containing all message‑specific values

This design allows the GUI to generate input fields dynamically without hard‑coding field layouts.

### 1.13.3	8.3 Status Callbacks

The API provides a status callback function pointer. The GUI registers this callback to receive live status messages for: - Initialization - File binding - Transmission start/stop - RINEX output enable/disable

This creates a responsive, operator‑friendly user experience.

## 1.14	9. Build, Deployment, and Runtime Workflow

### 1.14.1	9.1 Build Process

The project uses a Makefile with targets for: - hercules_transmitter.exe (console app) - hercules_transmitter.dll (GUI integration) - tests (validation tools)

Build highlights: - Cross‑platform GCC/Clang support. - Windows builds use Winsock (ws2_32). - DLL builds include all core modules, encoders, and orbit models.

### 1.14.2	9.2 Environment and Dependencies

- C compiler: GCC/MinGW on Windows, GCC/Clang on Linux/macOS.

- Python: 3.8 or later.

- GUI library: PyQt6.

- Build tools: GNU Make (optional; direct GCC build also supported).

### 1.14.3	9.2 Runtime Workflow

- Build DLL and executables.

- Launch GUI.

- Load a RINEX input file.

- Start transmission.

- Connect a TCP client (telnet or custom).

- Edit and resend messages as needed.

### 1.14.4	9.3 Operational Profiles

- Console‑only mode: Suitable for scripted or automated tests.

- GUI mode: Recommended for interactive editing and demonstrations.

- Batch simulation: Use a fixed duration and RINEX output to generate test datasets.

### 1.14.5	9.4 End‑to‑End Example Walkthrough

This example demonstrates a typical workflow used during the internship:

- Build the DLL

  - Compile the project and confirm that hercules_transmitter.dll is generated.

- Launch the GUI

  - The GUI initializes the DLL and starts the TCP server on port 23.

- Load Input File

  - Select data/parsing_file.25B.

  - Confirm successful parsing and message count.

- Start Transmission

  - Use default settings (all satellites, all fast corrections).

  - Verify that the status indicates “Transmitting.”

- Connect Client

  - Use telnet or netcat to connect to localhost:23.

  - Observe the continuous SBAS frame stream.

- Edit MT9 Message

  - Navigate to the Message Editor tab.

  - Select satellite 128 and MT9.

  - Adjust xg_ecef and iodp values.

  - Apply updates and enable increment mode.

- Continuous Single‑Message Mode

  - Start continuous MT9 transmission at a 1‑second interval.

  - Confirm the first frame matches exact values.

  - Confirm subsequent frames show LSB‑based increments.

- Enable RINEX Output

  - Choose an output file path and enable RINEX logging.

  - Verify the output file contains transmitted frames and header.

This workflow validates the full lifecycle: parsing, editing, scheduling, transmission, and output logging.

## 1.15	10. Testing and Validation

### 1.15.1	10.1 Unit and Integration Tests

Test harnesses validate decoding, CRC handling, and pipeline correctness at a message‑type level. These tests focus on: - Bit‑accurate decoding of payload fields - CRC‑24Q validation correctness - RINEX parsing accuracy and packet assembly

### 1.15.2	10.2 Manual Validation

- TCP client used to verify broadcast messages.

- GUI field editor used to change MT9 values and confirm re‑encoding.

- Continuous transmission verified with scheduler timing.

### 1.15.3	10.3 Test Scenarios

- End‑to‑End Pipeline

  - Input: RINEX file with mixed MT types.

  - Expected: All valid packets decoded and scheduled.

  - Result: Successful parsing and broadcast of multiple types.

- Field Update Validation

  - Input: MT9 message with edited position and IODP.

  - Expected: First transmission matches exact values; subsequent transmissions increment.

  - Result: Verified increment behavior in continuous mode.

- Orbit Model Override

  - Input: TLE or Statistical model selection.

  - Expected: MT9 position/velocity values replaced by predicted values.

  - Result: Observed stable, repeatable changes in GEO navigation data.

- RINEX Output Logging

  - Input: Enabled RINEX output with template file.

  - Expected: Output file contains header and transmitted frames.

  - Result: Verified output matches template format and includes MT63 fillers.

### 1.15.4	10.4 Observations

- Decoder outputs are consistent with encoder outputs, preserving round‑trip accuracy.

- Scheduler timing aligns with RTCA interval tables.

- GUI remains responsive during continuous transmission due to threading.

## 1.16	11. Performance and Reliability Considerations

- C backend ensures high throughput and deterministic timing.

- Non‑blocking sockets prevent transmission loop stalls.

- CRC validation ensures data integrity.

- Dynamic memory growth minimizes overhead during parsing.

- Auto‑increment system simulates realistic data variation.

### 1.16.1	11.1 Time Complexity

- Parsing: O(n) in file size.

- Decoding: O(m) for each packet payload (fixed length).

- Scheduling: O(64) per tick (fixed number of message types).

- Broadcast: O(c) per tick, where c is client count.

### 1.16.2	11.2 Memory Usage

- Message array grows proportionally to number of valid packets.

- Each message stored as a typed struct (fixed size).

- Growth strategy minimizes reallocation overhead.

### 1.16.3	11.3 Reliability Features

- CRC‑24Q validation eliminates corrupt packet transmission.

- Non‑blocking sockets prevent UI or scheduler stalls.

- Graceful client disconnect handling.

- Input validation in both C and GUI layers.

### 1.16.4	11.4 Security Considerations

- TCP server is designed for local or trusted network use.

- No authentication is implemented (future enhancement recommended).

- Inputs are validated to avoid crashes on malformed data.

## 1.17	12. Challenges and Resolutions

### 1.17.1	Challenge 1: Parsing RINEX Variants

Different file formats required robust parsing with flexible token handling. This was solved by creating portable tokenization helpers and validating SBAS packet lines.

### 1.17.2	Challenge 2: Correct Bit‑Level Encoding

SBAS messages require strict bit alignment. The solution was to implement explicit MSB‑first bit writers and match decoder offsets precisely.

### 1.17.3	Challenge 3: GUI Responsiveness

Transmission loops could freeze the GUI. The solution was to move transmission processing to a background thread while keeping UI updates on the main thread.

### 1.17.4	Challenge 4: Maintaining Feature Parity

The GUI needed to match console features (editing, scheduling, single‑message transmission). This required: - A complete DLL API mirroring console functionality. - JSON‑based serialization for message fields. - Careful mapping of field names to typed structures.

### 1.17.5	Challenge 5: Orbit Model Integration

Orbit model predictions needed to override existing MT9 fields without breaking encoding. This was solved by applying overrides just before encoding, ensuring clean separation between stored messages and generated outputs.

### 1.17.6	Challenge 6: Usability of Complex Fields

Many SBAS messages contain arrays or nested fields. The GUI flattened nested data into dotted field names and implemented a grid editor to make updates manageable for users.

## 1.18	13. Future Enhancements

- Unit tests for all message types.

- Per‑field validation in GUI.

- Advanced visualization dashboards for real‑time monitoring.

- Full parity of IOD synchronization in GUI mode.

- Export tools for CSV and analytics.

Additional proposals: - Automated regression tests comparing decoder and encoder round‑trips. - GUI profile management (save/load configurations). - Real‑time message statistics (throughput, message type distribution). - Enhanced security with authentication tokens for TCP clients. - Direct integration with GNSS simulation environments.

## 1.19	14. Conclusion

This internship delivered a complete SBAS extraction and transmission system with both backend and frontend implementations. The project demonstrates professional software engineering practices: modular architecture, clear separation of concerns, full documentation, and robust testing. The resulting system is suitable for research, simulation, and operational analysis of SBAS messages.

## 1.20	15. References

- RTCA DO‑229D: Minimum Operational Performance Standards for GPS/SBAS

- RINEX 2.10 SBAS Broadcast Format

- Internal Project Documentation (DOCS folder)

## 1.21	16. Appendix A: Public DLL API Summary

Key DLL functions include: - hercules_init / hercules_cleanup - hercules_bind_file / hercules_get_message_count - hercules_start_transmission / hercules_stop_transmission - hercules_process_transmission - hercules_get_message / hercules_update_message_field - hercules_send_message_once - hercules_start_single_message_tx / hercules_stop_single_message_tx - hercules_set_orbit_model_tle / hercules_set_orbit_model_statistical - hercules_configure_rinex_output

### 1.21.1	16.1 API Reference (Functional Summary)

| Category | Function | Purpose |
| --- | --- | --- |
| Initialization | hercules_init(port) | Initializes TCP server and core state |
| Initialization | hercules_cleanup() | Cleans up sockets and internal state |
| File Binding | hercules_bind_file(path) | Parses and binds a RINEX file |
| File Binding | hercules_get_message_count() | Returns count of decoded messages |
| File Binding | hercules_get_bound_file() | Returns currently bound file path |
| Transmission | hercules_start_transmission(filter, fc_count, duration) | Starts scheduled transmission |
| Transmission | hercules_stop_transmission() | Stops transmission |
| Transmission | hercules_process_transmission() | Executes one scheduler step |
| Transmission | hercules_is_transmitting() | Returns transmit status |
| Clients | hercules_get_client_count() | Returns number of connected clients |
| Message | hercules_get_message(sat, mt, out_json) | Returns message fields as JSON |
| Message | hercules_update_message_field(sat, mt, field, value) | Updates a single field |
| Message | hercules_send_message_once(sat, mt) | Encodes and sends a single message |
| Message | hercules_start_single_message_tx(sat, mt, interval) | Starts continuous single‑message mode |
| Message | hercules_stop_single_message_tx() | Stops single‑message mode |
| Increment | hercules_enable_field_increment(sat, mt) | Enables auto‑increment rules |
| Orbit Model | hercules_set_orbit_model_tle(tle, map) | Enables TLE model |
| Orbit Model | hercules_set_orbit_model_statistical(sat) | Enables Statistical model |
| Orbit Model | hercules_disable_orbit_model() | Disables orbit overrides |
| RINEX Output | hercules_configure_rinex_output(out, template) | Enables/disables output writer |
| Status | hercules_get_last_error() | Returns last error string |
| Status | hercules_get_transmission_stats(buf, size) | Returns transmission stats |
| Status | hercules_set_status_callback(cb) | Registers status callback |

## 1.22	17. Appendix B: Message Types and Update Intervals

(Intervals follow RTCA DO‑229D)

| MT | Description | Interval (sec) |
| --- | --- | --- |
| 1 | PRN Mask | 120 |
| 2‑5 | Fast Corrections | 6 |
| 6 | Integrity | 6 |
| 7 | Degradation | 120 |
| 9 | GEO Navigation | 120 |
| 10 | Degradation Parameters | 120 |
| 12 | UTC Data | 300 |
| 17 | GEO Almanac | 300 |
| 18 | IGP Mask | 300 |
| 24 | Mixed Corrections | 6 |
| 25 | Long‑Term Corrections | 120 |
| 26 | Ionospheric | 300 |
| 28 | Covariance | 120 |
| 63 | Null Message | As needed |

## 1.23	18. Appendix C: Project Directory Structure

SBAS_Extraction/
├── src/
│   ├── hercules_transmitter.c
│   ├── hercules_api.c
│   ├── encoder.c
│   ├── rinex_reader.c
│   ├── File_Parsing.c
│   ├── tx_server.c
│   ├── tx_source.c
│   └── GUI/
│       ├── main.py
│       ├── controller.py
│       ├── main_window.py
│       └── field_editor.py
├── DOCS/
└── data/

## 1.24	19. Appendix D: Message Field Reference (Summary)

This appendix provides a summary of the most commonly edited fields per message type. It is intended as a quick reference for users editing messages in the GUI.

MT1 – PRN Mask - iodp

MT2–MT5 – Fast Corrections - iodf, iodp - sat_corrections[i].fast_correction - sat_corrections[i].udrei

MT6 – Integrity Information - iodf2, iodf3, iodf4, iodf5 - udrei[i]

MT7 – Degradation - t_lat, iodp - udre_ai[i]

MT9 – GEO Navigation - t0, ura - xg_ecef, yg_ecef, zg_ecef - xg_rate, yg_rate, zg_rate - xg_accel, yg_accel, zg_accel - ag_f0, ag_f1

MT10 – Degradation Parameters - B_rrc, C_ltc_lsb, C_ltc_v1, I_ltc_v1 - C_ltc_v0, I_ltc_v0, C_geo_lsb, C_geo_v - I_geo, C_er, C_iono_step, I_iono, C_iono_ramp - rss_udre, rss_iono, C_covariance

MT17 – GEO Almanac - t0 - geo[i].data_id, geo[i].prn_number, geo[i].health_and_status - geo[i].x_g, geo[i].y_g, geo[i].z_g - geo[i].xg_dot, geo[i].yg_dot, geo[i].zg_dot

MT18 – Ionospheric Grid Mask - number_of_bands, band_number, iodi

MT24 – Mixed Corrections - iodp, block_id, iodf - fast_corrections[i], udreis[i]

MT25 – Long‑Term Corrections - half1.* and half2.* (velocity_code + per‑satellite fields)

MT26 – Ionospheric Delay - band_number, block_id, iodi - igp_vertical_delay[i], givei[i]

MT28 – Covariance - iodp - sat[i].prn_mask, scale_exponent - sat[i].E11..E44 and sat[i].E12..E34

## 1.25	20. Appendix E: Operational Checklist

Before Transmission 1. Build DLL and ensure it exists in the project root. 2. Launch GUI and confirm initialization on the configured TCP port. 3. Load a valid RINEX SBAS broadcast file. 4. Verify that decoded message count is non‑zero.

During Transmission 1. Confirm status shows “Transmitting”. 2. Check client count for active connections. 3. Monitor system log for errors.

Editing Messages 1. Select satellite ID and message type. 2. Modify fields and apply updates. 3. Enable continuous mode if incremental behavior is required.

After Transmission 1. Stop transmission. 2. Disable RINEX output if enabled. 3. Close the application cleanly to release sockets.

End of Report